DEPARTMENT OF ADSE

CSE434P-Computer Networks

PROJECT REPORT

B. Tech - Computer Science and Engineering

(AIML)

Submitted By:

Abel Alexander - 2462004

Annmarie Vinish – 2462041

Darain Brit A – 2462060

4BTCSAIML A

Abstract

Natural disasters frequently damage conventional communication infrastructure, disrupting coordination between victims and rescue teams. This project presents a simulation of an Emergency Communication Network using a drone-based mesh architecture. The system is implemented in two layers:

- Cisco Packet Tracer simulation demonstrating dynamic routing and network redundancy using OSPF.

- Python-based distributed simulation implementing priority-based message transmission using priority queues.

The system ensures self-healing communication, multi-hop forwarding, and prioritization of life-critical emergency messages. This project integrates networking principles and distributed systems concepts to model disaster-resilient communication.

Background

During disasters, communication systems such as cellular towers may fail. Drone-based networks provide temporary communication infrastructure where mobile nodes form a mesh network to relay emergency messages. This project simulates such a system at:

- Network Layer (Cisco Packet Tracer)

- Application Layer (Python Distributed System)

Problem Statement

The project addresses:

- Maintaining communication when infrastructure fails.

- Dynamically rerouting traffic if a node fails.

- Prioritizing emergency messages.

- Simulating distributed multi-hop communication.

Objectives

- Design a drone mesh topology.

- Implement OSPF dynamic routing.

- Add redundant backup links.

- Simulate priority-based message transmission.

- Demonstrate self-healing network behaviour.

Cisco Packet Tracer Implementation

Network Topology

The drone nodes are represented by routers connected in a mesh topology with a backup link.

Topology:

PC → R1 → R2 → R3 → R4 → Server
Backup Link: R1 ↔ R3

IP Addressing and OSPF Configuration

Each router interface is assigned a unique subnet. OSPF routing protocol is enabled on all routers to allow automatic path discovery and rerouting.

Dynamic Routing and Self-Healing

When Router R2 is shut down, OSPF recalculates the shortest path and reroutes traffic via the backup link.This demonstrates:

- Fault tolerance

- Redundancy

- Self-healing communication

Quality of Service (QoS)

QoS policies were configured using:

- Access Control Lists (ACL)

- Class Maps

- Policy Maps

- Service Policies

Traffic classes:

- HIGH – Medical emergency

- MEDIUM – Urgent assistance

- LOW – Default traffic

Due to limitations of Cisco Packet Tracer, full real-time queue scheduling under congestion cannot be visually demonstrated.

Python Distributed Mesh Simulation

System Architecture

The Python-based system consists of:

- sender_client.py – Victim device simulator

- node.py – Drone mesh node

- base_station.py – Rescue control center

Messages are forwarded dynamically between nodes until reaching the base station.

Priority Queue Implementation

Each drone node maintains a priority queue:

- HIGH → Priority value 1

- MEDIUM → Priority value 2

- LOW → Priority value 3

Messages are processed based on priority before forwarding.

Dynamic Forwarding

Each node maintains a neighbour list and attempts to forward messages to the next available node. If a node fails, alternate forwarding can occur.

Base Station Processing

The base station:

- Receives messages

- Displays structured output

- Stores data in messages.json

- Tracks priority statistics

Results

The system successfully demonstrated:

- Multi-hop communication

- Dynamic routing with automatic rerouting

- Redundant backup path activation

- Priority-based message processing

- Successful end-to-end transmission

Cisco Packet Tracer validated routing and redundancy behavior.

Python simulation validated priority scheduling and distributed forwarding logic.

Conclusion

The project successfully models a drone-based emergency communication network at both networking and application layers.

Key achievements:

- Dynamic OSPF routing

- Redundant topology design

- Priority-based emergency transmission

- Self-healing network behavior

- Distributed system simulation

The integration of Cisco routing concepts and Python distributed systems creates a comprehensive disaster-resilient communication model.

Future Scope

- Real wireless mesh implementation

- Cloud-based monitoring dashboard

- AI-based route optimization

- Integration with IoT distress sensors

- SDN-based centralized control

- Real drone hardware deployment